Specifications for Freeze-Resistant Stock Tanks/Fountains

October 1, 2003

Scope:  These specifications set forth the minimum technical requirements for selection and installation freeze-resistant stock tanks/fountains.

Product and installation specifications:

- Tanks/Fountains shall be enclosed, fully foam-insulated and completely sealed in impact-resistance polyurethane

- Tanks/Fountains shall have elliptical or flap closures that tip easily so animals can drink without resistance

- Tanks/Fountains shall be sized in accordance with manufacturer’s specifications for the type and number of animals for which it will be used

- Tanks/Fountains closures shall be insulated by foam insulation or dead air space and seal the opening completely

- Tanks/Fountains water supply shall be hard-piped underground and stubbed up into the insulated portion of the fountain

- Tanks/Fountains shall not contain any type of electric heat

- Minimum five-year manufacturer defect warranty

________________________________________

q:\te\rtf\specifications\freeze-resistant stock tanks\freeze-resistant stock tanks.doc

These specifications were adapted from Oregon Trail Electric Consumers Cooperative, Inc.